## Indoor Experiment Workflow

### Lab

- Connection

  - Connect the FSC-UTIAS laptop to Helson’s Wi-Fi.

  - Power on the Orin (make sure it is also connected to Helson’s Wi-Fi), then connect to it via SSH in VS Code.

- Test the Micro XRCE-DDS Agent

  - cd ~/dev_ws/src/fsc_autopilot_ros2

./scripts/indoor_exp/start_microxrce_agent.sh

  - ros2 topic hz /uav_0/fmu/out/sensor_combined

  - ros2 topic hz /uav_0/fmu/out/vehicle_attitude

  - Verify the topic frequencies on the laptop as well.

- After stopping the MicroXRCEAgent, reboot PX4 through QGroundControl.

[Orin]

- ~/dev_ws/src/fsc_open_manipulator/scripts/indoor_exp/start_open_manipulator_inverted_position.sh --no-gui

[Laptop - Shiqi ThinkPad]

- ~/ros2_ws/src/fsc_open_manipulator/scripts/indoor_exp/start_ground_station_inverted.sh

- Build the packages

  - Build fsc_autopilot_ros2

cd ~/dev_ws

colcon build --packages-select fsc_autopilot_ros2

source install/setup.bash

  - Build fsc_autopilot_ros2 when the Orin does not have Internet access.
cd ~/dev_ws

colcon build --cmake-args -DBUILD_TESTING=OFF --packages-select fsc_autopilot_ros2

source install/setup.bash

### Dome

- Connection

- [Laptop]

  - Start MOCAP
~/source/fsc_autopilot2_ws/src/fsc_optitrack_processor_ros2/scripts/launch_optitrack.sh 192.168.2.230 uav_0

  - Start the drone ground station
cd ~/source/fsc_autopilot2_ws/src/ros2_ground_station_gui && python3 src/single_drone_ground_control.py

  - Start the robotic arm ground station
source ~/fsc_autopilot_workspace/install/setup.bash

ros2 run custom_gui joint_plot_inverted --ros-args \

-r __ns:=/uav_0/fsc_open_manipulator \

-p torque_constant:=1.81 \

-p joint4_axis:="0 0 1"

- [Orin]

  - Start the MicroXRCEAgent
~/dev_ws/src/fsc_autopilot_ros2/scripts/indoor_exp/start_microxrce_agent.sh

  - Start the flight stack

~/dev_ws/src/fsc_autopilot_ros2/scripts/indoor_exp/start_geometric_direct_actuation_stack_t650.sh

~/dev_ws/src/fsc_autopilot_ros2/scripts/indoor_exp/start_geometric_direct_actuation_stack_t650_aerial_manipulator.sh

- Start the arm stack - position mode

~/dev_ws/src/fsc_open_manipulator/scripts/indoor_exp/start_open_manipulator_inverted_position.sh \

--no-gui namespace:=uav_0/fsc_open_manipulator

- Record a ROS 2 bag

ros2 bag record -o debug_recording_geometric_direct_$(date +%Y%m%d_%H%M%S) \

/uav_0/fsc_autopilot_ros2/position_controller/reference \

/uav_0/state_estimator/enu/imu/data \

/uav_0/state_estimator/local_position/odom \

/uav_0/state_estimator/local_position/optitrack_euler \

/uav_0/state_estimator/local_position/pixhawk_euler \

/uav_0/fmu/out/vehicle_attitude \

/uav_0/fmu/out/sensor_combined \

/uav_0/fmu/out/vehicle_status_v1 \

/uav_0/fsc_autopilot_ros2/geometric_direct_actuation/mode \

/uav_0/fsc_autopilot_ros2/controller_type \

/uav_0/fsc_autopilot_ros2/position_controller/state \

/uav_0/fsc_autopilot_ros2/position_controller/ude \

/uav_0/fsc_autopilot_ros2/attitude_controller/state \

/uav_0/fsc_autopilot_ros2/geometric_direct_actuation/geometric_control_debug \

/uav_0/fsc_autopilot_ros2/attitude_setpoint_debug \

/uav_0/fmu/in/vehicle_attitude_setpoint \

/uav_0/fsc_autopilot_ros2/geometric_direct_actuation/motors_debug \

/uav_0/fmu/in/actuator_motors \

/uav_0/fmu/in/vehicle_thrust_setpoint \

/uav_0/fmu/in/vehicle_torque_setpoint \

/uav_0/fmu/in/offboard_control_mode \

/uav_0/fsc_autopilot_ros2/vehicle_info \

/uav_0/fmu/out/vehicle_odometry \

/uav_0/fmu/out/estimator_status_flags \

/uav_0/fsc_open_manipulator/joint_states \

/uav_0/fsc_open_manipulator/position_controller/smoothed_reference_joint_trajectory

- Add the ROS 2 topics of the robotic arm.

Connect to the Orin via SSH:

- ssh jetson@192.168.50.107

param set SYS_HAS_MAG 0

param save

reboot